CS 437 Lab 5 Report

Frank Salamone (frankns2)

Video Link:

https://youtu.be/qUzcncA1L_g

GitLab Link:

Solo

Step 2 Design Your Device

2a. Questions and Design

The overall goal of this work is to simulate the activity of IoT devices deployed on animals in the African savannah.  Information gleaned from these sensors would be used to help in the conservation of zebras.  To this end, we want to study the health of the zebra population and gain an idea of the movements and interactions of these interesting, social animals.

1.  In our study of zebras, sensors affixed to the animal would need to determine information about the physiology and health of the animal, characteristics of the environment, and movement of animals through the environment.  Measuring the animal’s pulse rate may reflect its activity and/or anxiety level.  If a zebra is chased by a lion, its pulse will probably go up.  Pulse oximetry could be of some help, but if there is any analogy to the physiology of humans, oxygen saturation really won’t go down until the animal is in extremis.  Temperature and humidity gives us an idea of the animal’s environment to gauge its physiologic reactions.  Differences in behavior may be seen with differences in these variables over the short term, and with climate change, over the longer term.  GPS data can help us track the movement of the animals.

2.  There are a number of IoT communications protocols that balance power consumption, radio range, limited bandwidth, and intermittent connectivity.  As we saw in Lab 4, Amazon Greengrass Hub provides a central IoT device that can store data when connectivity to the cloud is interrupted.  Cell phone service is probably limited on the savannah.  One protocol that may be useful is a gossip protocol where each device tries to tell anyone that will listen its stored information.  As animals pass each other they can send this data on its way until an animal comes close enough to a base station.  This protocol would use a low power transmitter because the animals would be close when the data was transmitted.

3.  Given that these devices are deployed for long periods of time, and cannot be recharged, it would be critical that any code be optimized for low power consumption.  Overall, the code will loop through a series of data collection, storage and transmission steps.  These steps may change over time to optimize for low power consumption.

4.  I would put the device around the animal’s neck.  This seems to be customary.  I would imagine that this location would be difficult for the animal to get at with its mouth or hooves.  The neck provides access to the large vessels, giving a good idea of core temperature, and may make pulse oximetry more convenient.  The neck moves less relative to other parts of the body (e.g. the legs) providing a more stable base for the sensors.

5.  Any device that we attach to an animal traversing the African savannah is exposed to a harsh environment.  The animal’s surroundings are hot, dry, and dusty.  The animal we intend to study, the zebra, moves quickly and can come into contact with the environment by rubbing itself against other zebras or inanimate objects jostling our device.  Attaching the device to the zebra is expensive, time consuming, and not without risks.  The device would have to be light so that it did not decrease the fitness of the animal or injure it.  The device would need to be waterproof as zebras traverse streams and, on occasion, are rained on.

Device sketch:

2c Device Construction

Below, please find screen captures of my devices.   Construction of the devices was straightforward.  I used a number of devices in this lab.  I first tried to use the All in One circuit shown below.  This circuit produced log files perfectly but did not print out data during the simulation.    This can be seen in the image from the simulation below where the data box is empty.  I found that if I used multiple individual sensors, the data printed out in the simulation box without any problems.  I therefore on many of the simulations with multiple Arduino circuits such as Just GPS as seen below.

All in One circuit diagram:

Just GPS circuit:

Below, please find the serial output from the All in One circuit diagramed above.  I assume that the data output represents sensor readings from the device sitting in a room somewhere.  A temperature of almost 25 Celsius might be a little hot.  A humidity reading of 5 might represent 50% humidity, which would be normal room humidity.  The oxygen percentage and heart rate are zero because it is not attached to an animal.  These sensors show appropriate data in the wildlife simulation.

I ran into a few difficulties with the simulator.  Sometimes I would get an error 502.  I assume that this is a bad gateway error.  Sometimes the website times out, particularly with longer programs.  To counter this, I eliminated some of the extraneous components on my device such as the indicator LEDs.  These shorter programs, the simulator was less likely to time out during submission.

My code for device the All in One circuit:

Adafruit_GPS GPS;

PulseOximeter pox;

int val = 0;

void setup() {

pinMode(2, INPUT);  //Interrupt for vital sensor

pinMode(7, INPUT);  //Humidity

pinMode(8, INPUT);  //Sound

pinMode(A1, INPUT); //Temp

pinMode(A2, INPUT); //PPM

Serial.begin(9600);

}

void loop() {

if (Serial.available() > 0) {             //Read the GPS and print it out over serial line

string gpsResult = GPS.read();

Serial.println(gpsResult);

} else {

Serial.println("No data 1");          //If nothig on serial input indicate it by printing no data

}

if (Serial.available() > 0) {       //Read the pulseox and heart rate and print it out over serial line

if (pox.begin()) {

string spO2_res = pox.getSpO2();

string hb_res = pox.getHeartRate();

Serial.println("");

Serial.println("Oxygen percentage: " + spO2_res + " Heart rate: " + hb_res);

}

} else{                                   //If nothig on serial input indicate it by printing no data

Serial.println("No data 2");

}

float temp = analogRead(A1) / 1023.0 * 5.0 * 100.0;  //Read the temperature off of the A-D input 1

Serial.println("Temperature: " + to_string(temp));

float h = digitalRead(7);                            //Read humidity

Serial.println("Humidity: " + to_string(h));

val = digitalRead(8);                                //Read sound

Serial.println("Sound: " + to_string(val));

int PPMResult = analogRead(A2);                      //Read the air quality off of the A-D input 2

Serial.println("PPM: " + to_string(PPMResult));

delay(200);

}

2c Extra Credit

I affixed a transmitter on a lion that broadcasted “Roar”.  The circuit for the receiver is as below.  The transmitter is similar.

In the screenshot below, I show the zebra receiving the “Roar” message from the transmitter affixed to a  lion.

Step 3 Deploy Your Device

The main subject of this lab is zebra conservation, so the majority of my sensors were deployed on zebras.  I also needed enough zebra data to determine if the zebras formed friends that they stayed close to.  There seemed to be about 8-14 zebras spawned in a world.  I wanted to balance having a good number of animals versus wasting resources tagging the animals.  Tagging 6 zebras seems like an adequate number.  I was also interested in the lions.  I tagged 2 lions.  I wanted to see if they hunted the zebras.  I figured that a small number was needed as they would seek out the zebras!

Below, see 4 sensors and a receiver attached to a zebra.

One bug that I found was that I didn’t see the pink markers on any of my animals as shown in the figure below.

Step 4 Analyze Your Data

1.  Is the zebra population healthy?

The health of an individual can be gauged by measuring physiologic variables and observing patterns of behavior.  If the animal has problems with its lungs, its oxygen saturation will be low.  If the animal has problems with its heart, its pulse rate will not be normal.  In the paper “Cardiac evaluation of anesthetized Grevy’s zebras” by Adin, et al, the authors found the mean heart rate of 19 anesthetized zebras was 67 +/- 10 beats per minute.  If our Zebras don’t have a heart rate around 70 at rest or during very light exercise, they may not be healthy.

We can see that when zebra 142797 is moving at a slow rate, it has a pulse of 40 to 90.  This is what we would expect.  When the zebra runs its heart rate increases.

Zebras also appear to be displaying normal patterns of behavior in that they travel at multiple speeds.  From our reading in the paper “Energy-Efficient Computing for Wildlife Tracking: Design Tradeoffs and Early Experiences with ZebraNet” by Juang, et al, have two peaks to a graph of their distance moved, implying two different speeds.  We see this in our data as shown below.

We can see that the zebra displays two speeds of 6 and 1 maybe corresponding to graze walking and fast moving modes of zebra travel.  The point diameter is proportional to the animal’s heart rate.  The heart rate is greater when the animal is running.

****Maybe put something in about temp, humidity, air quality

2.  Do zebras have enough room to move around?

This is a bit difficult to figure out given that I can’t find the units of measurement in our world.  Zebras can travel 48 miles in a day https://www.skindhuset.dk/upload_dir/shop/Zebra.pdf.  The lab quotes Robin Naidoo who states herds of zebra can trek 150 miles in 2 weeks (10.71 miles/day).  We are told that our territory is 50 square miles, or 7.07 miles on a side.  So, 7.07 miles is close to 10.71 miles, it seems reasonable that our zebras have a whole day’s walk on each side of our enclosure.  It seems like they have sufficient room.

3.  Did you see any poachers?

I did not see any poachers.

4.  CDF of zebra movement.  We can see that there is a big jump in the cumulative distribution at speeds of 0, 1 and 6.  This is what I would expect given the second graph in the answer to the first question that shows speeds of 6 and 1.

5.  Do zebras make friends.  Are there any pairs that stay together?  I do not think so.  I tried to figure this out in a couple of ways.  First, I looked at the map for an extended period of time and could se no pattern to the zebras’ congregation.  Second, I tried to find zebras that started close together.  I then did position plots looking at these zebras, and they did not seem to stay together over time.

6.  What locations do zebras tend to congregate at?  Why do they tend to go there?

Zebras seem to go around the water in the center of the map.  I think they go there and concentrate there to drink.

7.  Are there any locations that zebras avoid?

It seems that zebras avoid the upper left portion of the map.  There are more trees in this region.  Maybe zebras prefer grasslands.  In addition, zebras don’t seem to go in the lower left where there are hills.

I ran the simulation with 11 zebras for almost a day, and my theory about the animals’ avoiding trees in the upper left corner is less strong.  They seem to avoid the perimeter of the habitat.  The left lower corner remains unvisited.

One additional question:

Are there any places where lions congregate?

The lions seemed to congregate in the center of the map near the water and near the zebras.  It seems like the lions congregated near the center of the map more than the zebras.  Maybe the zebras needed to graze (so, away from the water), while the lions just needed high concentrations of zebras (near the water).

I did find some evidence that lions stalk zebras.  In the plot below, we can see the lion in blue following the zebra and then crossing its path.